| SOR FOR TOPOGRAPHIC CONTOUR SURVEY AND ASSOCIATED CIVIL JOBS |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
| S. No. | ITEM CODE | DESCRIPTION | QTY | UOM | QUOTED PRICE/UOM | TOTAL QUOTED PRICE |  |
| 1 | 01CIVIL | Constructing 300mm x 300mm x 1000mm size BOUNDARY PILLARS in plain cement concrete of M20 grade with 20mm and down size graded stone aggregate with 500mm below ground level and 500mm above ground, including earthwork in excavation, backfilling and spreading surplus earth, shuttering, tamping, ramming, vibrating, curing and finishing the top and sides rough or smooth with pillar number as directed or specified with all labour and materials etc. complete as per specifications and directions of Engineer-in- Charge . | 510 | Each |  |  |  |
| 2 | 02CIVIL | Constructing 1500mm x 1500mm x 2000mm size PERMANENT PILLARS plain cement concrete of M-20 grade with 20mm and down size graded stone aggregate with 1000mm below ground level and 1000mm above ground including embedding steel plate 150 mm x 150 mm x 10 mm thick with two nos. anchor bars of 12mm dia. 150 mm long for marking permanent references points including earthwork in excavation, backfilling and spreading surplus earth, shuttering, tamping, ramming, vibrating, curing, and finishing the top and sides rough or smooth as directed or specified with all labour and materials etc. complete as per specifications and directions of Engineer-in-Charge . | 4 | Each |  |  |  |
| 3 | 03survey | Conducting topographical ground/land survey in the specified area by Aerial Photogrammetry, covering details of all physical features like Ponds, wells, pits, Trees, Roads etc. including establishing high flood level with source of information, identification of any monument/ historical structure/ wild life sanctuary in the vicinity, identifying and locating the storm water disposal point with levels, including providing in tabulation girth wise no. of trees etc. as specified, plotting and developing topographical contour maps with 0.5 contour interval, providing spot levels, ditigal terrain model, 3D image, ortho-photo etc. showing true north including establishing survey reference stations, key plan, carrying out DGPS survey at minimum 4 locations, establishing ground control points etc. including carrying out stereo photogrammetry processing, including all equipment, instruments, personnel as per requirement to carry out survey work etc., complete in all respects as per drawings, specifications and directions of Engineer-inCharge including submitting hard copies of drawings/map and supply of electronic files. | 400 | acre |  |  |  |
| 4 | 04survey | Preparation of list of boundary cordinates of the proposed area. | 510 | Each |  |  |  |
| 5 | 05Survey | Boundary cordinates marking with boundary mark up spray paint | 510 | Each |  |  |  |